Element 1- Aircraft Construction

1.1. Construction and Materials:

spars, truss, non-ferrous, formers, composites, monocoque, wings, semi-monocoque, helicopters, wings, ferrous

- ________________ construction is a reinforced shell that features a load bearing skin reinforced by a framework of structural members.

- ________________ metals are now largely confined to relatively small parts of the aircraft where high stress levels are encountered, such as the engine and landing gear components.

- A ________________ is a rigid framework that resembles the structural components of a bridge that is used in ________________, lightweight aircraft and ultra-light aircraft.

- A ________________ structure has no internal bracing, except for frames, formers and bulkheads that define its shape.

- ________________ metals are the prevalent material used throughout the aircraft and the airframe; with aluminium alloys, titanium and magnesium as the most common metals.

- ________________ construction is an eggshell type structure that relies entirely on the strength and shape of its skin to carry the loads.

- ________________ include fibreglass and carbon or graphite/epoxy laminates that have high strength to weight ratios, high corrosion resistance and high resistance to fatigue and impact damage.

- ________________ construction is used by most commercial and military aircraft, because it can absorb damage and fatigue loading.

- ________________ are normally constructed using monospar, two-spar, multi-spar or box beam principles, and have an internal structure that consists of ________________ and stringers running spanwise and ribs and ________________ running chordwise.

- ________________ metals are largely impractical to use in the construction of an airframe because of their low strength to weight ratios.

- ________________ are widely used in commercial and military fighter (F/A-18, F/A-22) to save weight as well as in stealth aircraft to evade radar.

1.2. Aircraft Components and Functions:

elevators, wings, ailerons, stabilisers, rudder, fuselage

- A _               _ is an aircraft's main body or central structural component that holds the crew and passengers and cargo and where the wings and tail are attached.

- The major function of the  _____________  in flight and on the ground is to hold people and to hold the plane together.

- The primary purpose of ___________ is to support the aircraft in flight, by developing lift when they move through the air.

- The  _________  accommodate the engines, main fuel tanks, main landing gear, and bomb racks and may even be fitted with external fuel tanks.

- The _                     keep the aircraft in a straight and level flight by preventing the aircraft from pitching or yawing in flight.

- are hinged surfaces attached to the trailing edge of the horizontal stabiliser that allows an aircraft to climb or dive in flight.

- The ___________ is a hinged surface attached to the vertical stabiliser to properly align the aircraft during turns in flight or to correct deviations to port or starboard during takeoff and landing on the runway / ground.

- are hinged surfaces on the wings that allows an aircraft to roll to port or starboard in flight.

1.3. Aircraft Components and Functions:

flaps, rotor, landing gear, spoilers, doors & panels, trim tabs, slats, air or speed brakes

- are small airfoils recessed in the trailing edge of a primary control surface that enable the pilot to neutralize any unbalanced condition that might exist in flight, without exerting any pressure on the control stick or rudder pedals.

- ____________ are normally mounted between the inboard end of the ailerons and the fuselage that provide additional lift during takeoff and reduce the stalling speed during landing.

- provide access to the aircraft and servicing points for repairs and maintenance and may be hinged or removable and have identification labels.

- are fitted to the front of the wing and perform a similar role to Flaps.

- The ___________  generates lift and propulsion using two or more symmetrically shaped blades or airfoils that are rotated at high speeds.

- ____________  are movable panels that are hinged to the sides or bottom of the fuselage or to the wings that are designed to rapidly reduce speed during a steep descent or when approaching a runway for landing.

- ____________  are mounted in the upper surface of each wing that are designed to disrupt the airflow to dissipate the lift during flight and slow down the aircraft after landing on the ground.

- The _                      assembly supports and distributes the weight of the aircraft during landing or while it is moving or stopped on the ground and it retracts after takeoff.