# Element 2 - Aircraft Flight Principles

2.1. Aircraft Speed and the Speed of Sound

lower, subsonic, transonic, speed of sound, Mach Number

- The ____________ is the ratio of the aircraft speed to the speed of sound that is used to measure the Aircraft Speed and Aerofoil flow in flight.

- _______________  aircraft speed has Mach numbers less than one.

- _____________ aircraft speed has Mach numbers from 0.8 to 1.2.

- The _______________ depends on temperature and is normally greater at the higher temperatures that usually occur at sea level.

- The speed of sound decreases at the _________ temperatures that usually occur at higher altitudes.

2.2. Aerofoils

camber, aerofoil, shape, chord line,  angle of attack, wings, lift, relative airflow, stall, drag, Venturi tube

- When a fluid flows through a ________________, the velocity of the fluid passing through the constriction increases while its pressure decreases.

- An ________________ is any surface that produces a reaction when air passes over it.  This includes aircraft wings, stabilisers, propellers and helicopter rotors.

- The ________________ is a straight line drawn from the centre of curvature of the leading edge to the trailing edge of an aerofoil.

- The ________________ is the angle between the aerofoil chord line and the relative airflow.

- The ________________ is the curvature of the upper and lower surface of an aerofoil.

- The ________________ is the surrounding airflow that is not affected by the aircraft's passage through it.

- ________________ are aerofoils that are shaped so that when they are moved through the air, the curved upper surface causes the air to accelerate, thereby reducing the air pressure.

- ________________ is the upward force that occurs when the air pressure is lower on a wing's curved upper surface relative to the higher air pressure below the wing.

- The ________________ and area of an aerofoil determines the amount of lift.

- As an aerofoil’s ________________ increases, the lift increases until it reaches maximum.  After which the aerofoil ________________ and the flow properties drastically change and the aerofoil loses lift, due to increased ________________.

2.3. Subsonic and Transonic Aerofoil Flow

standing wave, subsonic, sound waves, transonic, sonic boom, supersonic, wave drag, shock wave

- ________________ flights are when all the air passing over the aircraft is less than the speed of sound at the altitude.

- ________________ flights are when the air passing over the aircraft is a mixture of both subsonic and supersonic speeds.

- ________________ flow is often unsteady and the shock waves on the body surface may jump back and forth along the surface, thus disrupting and separating the flow over the wing surface.

- ________________ during transonic flight can be reduced by the use of thin airfoils, forward or backward swept wings, and low aspect ratio wings.

- ________________ flights are when all the air passing over the aircraft is greater than the speed of sound at the altitude.

- When an aircraft flies as fast as the speed of sound, the ________________ in front cannot move away from the aircraft and become like a ________________ that compresses in the front of the aircraft to form a ________________ that produces a ________________ due to the pressure differential between the shock wave and the surrounding air.

2.4. Forces on Fixed or Rotary Winged Aircraft

compressibility, drag, velocity, weight, steady flight, lift, form, equal, induced, wave, wing area, angle of attack, thrust, fixed,

- ________________ is the forward horizontal force produced by the power plant or propeller to overcome the force of the drag to move the aircraft through the air.

- ________________ is a backward horizontal force (aerodynamic resistance), caused by the disruption of airflow by the wing, fuselage and other protruding objects that oppose the thrust.

- ________________ drag depends on the shape of an aircraft.

- ________________ drag is additional drag caused by the generation of lift by the wings, which can be reduced through using winglets or vortex generators.

- ________________ drag results from shock waves produced when an aircraft approaches Mach 1, which can be reduced by using swept or delta wings in a supersonic Aircraft.

- ________________ is the downward vertical force of gravity resulting from the combined load of the aircraft, crew, fuel and cargo that opposes the lift.

- ________________ is the upward force produced by the dynamic effect of the air acting on the wing, perpendicular to the flight path and opposes the weight.

- The factors that affect lift include the wing geometry, ________________, ________________ of the aircraft and density, viscosity and ________________ of the air.

- Flaps slid backwards and slats moved forwards increase the ________________ significantly, thus significantly increasing the wings' lift.

- During ________________ (level, in a straight line and at a constant velocity), the sum of each set of opposing forces is equal to zero.  This does not mean that the thrust/drag set of opposing forces is ________________ to the lift/weight set of opposing forces.

- Normally for an aircraft to cruise at a ________________ altitude, the lift/weight opposing forces are greater than the thrust/drag opposing forces.

2.5. Forces on Rotary Wing Aircraft

upwards, perpendicular, same, downwards, backwards, drag, sideways, hover , tilt angle, equal, thrust, angle of attack

- Helicopters are different than fixed winged aircraft because they can move ________________, ________________, ________________, ________________ and ________________.

- When all four forces are ________________, a helicopter can hover.

- ________________ can be either a vertical or horizontal force produced by the ________________ of the rotor blades to overcome the force of the weight to hover or move upwards or overcome ________________ to move the aircraft through the air in any direction.

- Lift is produced by changing the ________________ of the rotating aerofoils or rotor blades.

- Lift can be in the ________________ direction as the thrust when a helicopter moves upward or at a direction ________________ to the thrust when it moves forwards, backwards or sideways.

2.6. Aircraft Axes and Control

elevator, centre of gravity, pitching, aileron, lateral, rudder, longitudinal, yawing, vertical, stability, directional, control, equilibrium, pitching, rolling, orientation

Words can be used more than once, if necessary.

- Aircrafts move in three dimensions, which make it necessary to control the attitude or orientation of a flying aircraft in all three dimensions. In flight, an aircraft rotates about its ________________.

- The ________________ axis passes through the centre of gravity of the wings; the ________________ axis passes through the centre line of the aircraft and the centre of gravity; and the ________________ axis passes vertically through the centre of gravity.

- The ________________ of an aircraft can be defined by the amount of rotation of the parts of the aircraft along these principal axes.

- ________________ is the tendency or lack of it, of an airplane to fly a prescribed flight condition, while ________________ is the ability of a pilot to change the airplane's flight conditions.

- A plane is in ________________ if lift equals weight, thrust equals drag and there are no net rotating moments acting on it.

- ________________ stability and control is about the lateral axis and is concerned with an airplane's ________________ motion that is controlled by ________________ movement.

- ________________ stability and control is about the longitudinal axis and relates to an airplane's ________________ motion that is controlled by ________________ movement.

- ________________ stability and control is about the vertical axis and relates to an airplane's ________________ motion that is controlled by ________________ movement.

2.7. Fixed Wing Aircraft Primary Flight Controls

rolling, elevators, yawing, pitching, ailerons, increases, bank, rudder, decreases

- As the aileron on one wing is deflected downward, the lift on this wing ________________ and as the aileron on the other wing is deflected upward the lift on this wing ________________.

- During take off the ________________ are used to bring the nose of the aircraft up to begin the climb.

- ________________ produce lateral stability by controlling the ________________ motion about the longitudinal axis of an aircraft; and are used to ________________ or turn an aircraft.

- The ________________ is a hinged surface attached to the vertical stabiliser that controls the position of an aircraft's nose during flight.

- During a banked turn, ________________ can be used to increase the lift by changing the attack of the wing and cause a tighter turn.

- The ________________ is NOT used to turn the aircraft in flight, but insures that the aircraft is properly aligned to the curved flight path during a banking manoeuvre.

- Elevators produce longitudinal or horizontal stability by controlling the ________________ motion about the lateral axis of an aircraft.

- The rudder produces directional stability by controlling the ________________ motion about the vertical axis of an aircraft.

2.8. Fixed Wing Aircraft Secondary Flight Controls

trim tabs, maintain, flaps, balance, slats, speed brakes, spoiler, help move

- ________________ and ________________ are designed to provide additional lift during takeoff and reduce the stalling speed during landing, so that lower speeds and shorter landing distances are possible.

- ________________ are small airfoils recessed in the trailing edge of the aileron, rudder or elevator control surfaces to________________ the airplane to fly in a stable cruise condition; ________________ the elevator, rudder and ailerons at whatever setting the pilot wishes without the pilot maintaining pressure on the controls and ________________ the elevators, rudder, and ailerons and thus relieve the pilot of the effort necessary to move the surfaces.

- When the ________________ are moved aft (toward the tail) and the ________________ moved forward, the wing surface area increases to increase the lift.

- ________________ are drag-producing surfaces that are deployed in flight when a rapid reduction in speed is required during a steep descent or when approaching a runway for landing.

- When the lift ________________ is deployed in flight, it disrupts the airflow to dissipate the lift.

- The ________________ and ________________ move along metal tracks within the wings and provide additional lift or drag by increasing the wing area and changing the airfoil shape.

- Many aircraft deploy ________________ and airbrakes to slow the plane down for landing and when on the ground.

2.9. Rotary Wing Flight Controls

cyclic stick, pitch, roll, tail, torque, yaw, rotate, dual-rotor, throttle, collective lever, direction and velocity, elevators, tilting, anti-torque

- The pilot uses the ________________ to control ________________ by tilting the main rotor forward or backwards to pitch the helicopter backwards or forwards.

- The pilot uses the ________________ to control the roll by ________________ the main rotor to port or to starboard to roll the helicopter to starboard or to port, respectively.

- A helicopter with a single driven rotor requires a ________________ rotor mounted on the helicopter's tail boom to produce sideways or horizontal thrust to counteract any ________________ reaction and provide directional control.

- The pilot's ________________ pedals that are located on the floor in the same place as a fixed-wing aircraft's rudder pedals control the ________________ about the vertical axis.

- By pushing a pedal a pilot can increase or decrease the horizontal thrust to ________________ the helicopter around its vertical axis.

- ________________ helicopters have a differential between the two rotor transmissions that can be adjusted by an electric or hydraulic motor to transmit differential torque to turn the helicopter.

- The ________________ control is a twist grip, located on the collective lever that controls the rotor power produced by the engine to insure that the rotor is operating at its design-specified RPM.

- The ________________ controls the main rotor pitch by increasing or decreasing the ________________ of the rotor blade to increase or decrease the lift.

- The cyclic stick is operated by the pilot's right hand to control the ________________ of horizontal flight.

- To achieve forward flight, the pilot moves the ________________ forward to tilt the rotor disc forward and at the same time increases the ________________ so that the helicopter does not sink.

- Synchronised ________________ may be fitted to the helicopter to improve passenger comfort and aid the control response in forward flight.

- The ________________ are inverted airfoils, linked to the cyclic control that drags the rear fuselage to a horizontal position as the helicopter moves forward.