## Element 3.3 & 3.4 - Aircraft Mechanical and Avionic Systems

# 4.1. Air Conditioning and Hydraulic Systems

selector valve , pressurization, humidity, actuator, simple to maintain, outflow,  engine bleed air, accumulator, ventilation, compression, air supply, lightweight, temperature, landing gear, brakes, primary flight controls, actuate

- Aircraft air conditioning systems are designed to supply human comfort requirements and insure that equipment operates effectively by controlling ________________, ________________ and ________________.

- The air conditioning system ________________ is high-pressure, high-temperature air that is supplied by blowers, compressors and ________________.

- The aircraft ________________ system governs the ________________ of conditioned air to maintain the required pressure within an aircraft structure to create an artificial pressurized environment.

- An aircraft hydraulic system uses fluid under pressure, as a medium, to transmit power to ________________, ________________, ________________, air brakes, bomb bay doors, flaps, cargo ramps and access ladders.

- Hydraulics work because liquids have a very high resistance to ________________ to provide positive, instant smooth control to ________________ hydraulic equipment.

- Hydraulic systems are ________________, easy to use, capable of accurate control, ________________ and are almost 100% efficient.

- When a ________________ is opened to operate a service, some pressurised fluid from the ________________ is forced into the ________________ to move it; and once the movement is complete, the engine driven pump replenishes the accumulator fluid used to initially move the actuator.

# 4.2. Pneumatic System

actuator, gas, relief, hydraulics, control, compressing, de-icing, compressor, instruments, air, cylinders, backup source, engine bleed air, compressor

- A pneumatic system utilises the energy generated by ________________ a ________________ to move an actuator or run a motor.

- Pneumatic systems are similar in principle to hydraulic systems and can be used to operate landing gear, brakes, primary and secondary flight controls, ________________  systems and aircraft ________________.

- On the ground, compressed ________________ is stored in storage ________________ until the air is required to actuate a system.

- During flight, the air is supplied from a ________________ that utilises supercharged air from the ________________ system.

- Normally, aircraft that utilise ________________ to operate equipment also have a cylinder of compressed air or nitrogen as a ________________ of power to lower landing gear or apply brakes, in the event of a hydraulic system failure.

- A simple pneumatic system includes a filter, an electrically or hydraulically driven ________________, a pressure ________________ valve, a reservoir, a drain valve, an inline filter, a gauge and ________________ valve.

- When the ________________ valve is opened to operate a service, pressurised air is forced into the ________________ port to extend or retract the actuator.

# 4.3. Fuel and Oxygen Systems

LOX, flexible, gas, sufficient, oxygen, rigid, engine bleed air, monitors, burns, masks, explosions, pressurisation, liquid, GOX, integral, controls, auxiliary, fuel pumps

- The aircraft fuel system insures that there is ________________ fuel for the aircraft to perform its operational role.

- The Fuel management system ________________ and ________________ the fuel flow to all of the engines.

- Fuel Tanks include ________________ tanks that are fitted either internally in the tank bays in the wing or in the fuselage or as external tanks, ________________ tanks that can be fitted into oddly shaped spaces, ________________ tanks that use the top and bottom wing skin and internal wing structure to provide the tank walls and side bulkheads or ________________ tanks (rigid or flexible) tanks that are fitted to an aircraft to increase the aircraft's operational range.

- Some aircraft use ________________ to transfer fuel from one tank to another to help balance the aircraft in flight, while others use ________________ to pump the fuel around.

- Above 3,000 m, ________________ levels must be maintained to sustain normal body functions.

- Normally, the aircraft ________________ system takes care of supplying oxygen, but in the event of an emergency, oxygen is supplied directly to the crew and passengers through ________________ from the Oxygen System.

- Oxygen can be stored either as a gas or as a ________________, but is always supplied as a ________________.

- Gaseous Oxygen (_______) is stored at extremely high pressures in order to allow large quantities to be held in relatively small GOX Bottles.

- Liquid oxygen (________) can be used to provide GOX when very small space is available, because one litre of ________________ produces 862 litres of ________________.

- Although LOX reduces the space required to store on board the aircraft, it also creates potential hazards such as ________________ from the extremely low temperature of -182°C, or ________________, if not properly vented.

# 4.4. Ice & Rain Protection and Sanitary Systems

electrical, de-icing, demist, water, lift, thermal, prevent, de-misting, drag, removal, anti-icing, disperse, waste, repellent, control, pneumatic, stability, repel

- The purpose of the aircraft ice and rain protection system is to ________________ and ________________ the formation of ice and rain on various parts of the aircraft.

- Ice is a real hazard to the safe operation of any aircraft because it decreases ________________, increases ________________, decreases propeller efficiency, freezes control surfaces, increases loading and reduces ________________.

- Aircraft ice and rain protection systems are used to ________________ the amount of ice on the aircraft by using anti-icing and de-icing, ________________ and remove rain from windscreen and ________________ the windscreen.

- ________________ prevents the formation ice on the aircraft by keeping its surface temperature above freezing, while ________________ breaks away the ice already formed on the surface of the aircraft.

- Three methods for ice control on an aircraft are ________________, ________________ and ________________.

- The rain protection system consists of the rain ________________ system, the rain ________________ system, windshield wipers and the windscreen ________________ system.

- An aircraft sanitary system includes ________________ treatment prior to supply and sewage ________________ treatment for commercial aircraft.

# 4.5. Landing Gear, Doors & Panels and Emergency Escape & Flotation

brake unit, standard, life jackets, nose wheel, canopies, wheel assembly, spread, independent braking, life rafts, absorb, tricycle, oleo assembly, braking, move, freight, landing gear, escape hatches

- The purpose of an aircraft's landing gear is to ________________ the weight of the aircraft on the ground, ________________ the high shock loads of landing, provide ________________ to slow the aircraft and enable the aircraft to ________________ on the ground.

- Normally an aircraft ________________ consists of a wheeled undercarriage, but it may be fitted with special equipment such as skis or floats for special use.

- Two common undercarriage systems include the ________________ undercarriage, common on wheeled helicopters and the ________________ undercarriage, commonly found on most aircraft.

- The landing gear undercarriage includes a ________________ that consists of the wheel hub, bearings, and tyre; ________________ that consists of the strut and shock absorber and the ________________.

- The two methods to steer an aircraft on the ground are ________________ steering normally used on large aircraft and ________________ steering normally used on small and medium sized aircraft.

- Aircraft doors and panels include the main access doors, ________________ doors, ________________, rear-loading ramps and access panels.

- For emergencies, all aircraft are designed with ________________ or doors for evacuation.

- ________________ are used by passengers after ditching and are normally stowed under seats.

- Inflatable ________________ are designed to hold the aircrew and passengers and keep them out of the water, thus preventing hypothermia.

4.6. Fire Detection & Protection Systems

protection, detect, sensors, light, overheat detectors, fire, alarm, see, extinguish, engine bays, warning, fire wire, disconnect

- ________________ aboard an aircraft is one of the worst-case scenarios imaginable, because the crew cannot ________________ most of the high-risk areas or may not be able to get to the fire.

- Aircraft fire detection systems use the following ________________ to activate the fire ________________ system: a continuous ________________ - high resistance fire wire is routed throughout the ________________ and ________________- temperature probes are placed in areas that are susceptible to fires and only ________________ a fire near the probe.

- Once a fire has been detected, the master fire warning ________________ and audible ________________ become activated.

- Fire ________________ systems are fitted to the engine bays to ________________ external engine fires; and once activated ________________ the engine from the fuel, oil, hydraulic and electrical systems.

4.7.   Avionics and Electrical Systems

batteries, AC, generate, ground power, electrical equipment, bus bars, DC, distribute, generators, battery, inverter

Avionics consist of all of an aircraft's ________________ that includes the electrical system, instruments, communications and radar.

The aircraft electrical system is designed to ________________ and ________________ electricity to all electrical systems such as radios, lighting and engine ignition.

Generators, batteries or an external ground power supply provide either direct current (_____) or alternating current (_____) to all aircraft electrical systems.

The _____ generation and distribution system supplies 28 Volts DC from either aircraft ________________ or ________________ through a series of protection devices to the aircraft ________________.

AC distribution systems use AC voltage that is converted from _____ voltage by an ________________.

When an aircraft is being serviced or the engines are started it may be necessary to use a ________________ unit, transformer rectifier unit or a ________________ cart to supply either 24-28 V_____ or

115 V_____.

4.8. Instrument Systems and Flight Instruments

skidding, air speed indicator, attitude, engine, turn and slip, altimeter, displays, navigation, slipping, flight, vertical speed indicator, dials, horizontal situation, position and pressure

Instrument systems display vital information on ________________ and computer ________________located in the cockpit to help the aircrew fly the aircraft safely.

4.8.1.Typical aircraft instrument systems include the following:

a. ________________ instruments that include the airspeed indicator, altimeter and direction indicator.

b. ________________ instruments that measure the quantity and pressure of fuel and oil, manifold pressure of gases, rpm and temperature.

c. ________________ indicators that provide information on the position of flaps and landing gear and pressure of vital systems.

d. ________________ instruments that provide accurate information that enables the pilot to travel along a definite course

4.8.2. Flight instruments:

a. ________________ senses air pressure from outside the aircraft to provide an indication of the aircraft's forward speed through the air.

b. ________________ provides an indication of the aircraft's height in feet or metres, either above sea level, ground level or a fixed reference point.

c. _________________ uses the changing air pressure outside the aircraft to provide an indication to the pilot of how fast the aircraft is climbing or descending.

d. ________________ indicator provides information about the rate of turn of the aircraft, and whether it is ________________ (dropping into the turn) or ________________ (sliding out of the turn).

e. ________________ indicator, sometimes known as Artificial Horizon, provides an indication of the aircraft's position in the sky (straight and level, left wing down or nose up etc) relative to the earth's surface.

f. ________________ indicator provides the pilot with an indication of the aircraft's compass heading, along with needles pointing towards any navigation aids that he has selected.

4.9. Engine and Position & Pressure Instruments

dolls eye, engine Oil Pressure, coloured lights, flap position, quantity, landing gear position, engine temperature oil, tachometer, engine temperature, pressure

4.9.1. Engine instruments:

a. ________________ indicator is a measuring device and an indicator or gauge located on the flight deck instrument panel that measures the fuel, GOX or LOX Quantity.

b. RPM indicator or ________________ indicates the speed of the crankshaft for a reciprocating or the speed of the main rotor assembly for a gas turbine engine.

c. ________________ indicator measures the temperature (generally in C) of the engine or the gases flowing through the engine using thermocouples.

d. ________________ Indicator measures the pressure (psi) of oil supplied to the engine and provides warning if there is any loss of pressure before it becomes critical.

e. ________________ Indicator - measures the temperature of oil within the engine.

4.9.2. Position and Pressure instruments:

a.The ________________ indicator indicates the position of the aircraft undercarriage with ________________ and a ________________ indicator that show the position for each wheel.

b. The ________________ indicator - indicates the position of the aircraft flaps.

c. ________________ indicators indicate the pressure for important systems throughout the aircraft and include measurements of fuel tank air pressure, hydraulic pressure, brake pressure, oil pressure, engine booster pressure and pneumatic pressure

4.10. Navigation Instruments

inertial navigation, horizontal situation indicator, satellites, gyros, radio, global positioning, VOR, antenna, TACAN, distance measuring, ADF

- ________________ navigation systems rely on radio signals transmitted from ground stations located at an airport being received by the aircraft radio receiver.

- The three main types used on an aircraft are VHF Omni-directional Ranging (______), Tactical Airborne Navigation (______) and Automatic Direction Findings (______).

- Radio navigation systems normally consist of an ________________, receiver and controller with outputs to the _____________________ (HSI).

- ________________ equipment works in conjunction with the VOR and TACAN to provide an indication of the distance on the HSI digital readout to a ground beacon from the aircraft.

- The ________________ system utilises a cluster of ________________ in orbit around the world to provide a constant indication of a position almost anywhere on the earth.

- ________________ systems, utilising ________________, are totally self-contained, requiring no external power.

4.11. Communication and Radar Systems

Doppler, navigation, direction, intercom, high frequency, radio detection and ranging, receives, very high frequency, distance, ultra-high frequency, antenna,

- ________________ (HF) radios, ________________ (VHF) / ________________ (UHF) radios and ________________ systems are used to communicate between the crew, between the aircraft and ground and between two aircraft.

- The ________________ radio is used to communicate over long distances, while the ________________ and ________________ radios are used to communicate over short to medium ranges (line of sight).

- The ________________ system is used for communication between crew members.

- RADAR (_______________________) operates by transmitting a high frequency beam from a directional ________________ to a target and measuring the return beam reflected back to the antenna to determine the ________________ and ________________ of an object.

- The radar ________________ system is typically a ________________ system that uses a downward firing radar beam to provide an indication of the aircraft's ground speed and sideways movement across the ground.